No internet connection when 4G LTE router in 4G mode with SIM card inserted

First, we need to login in the management UI, ‘Open browser’ -- ‘Type 192.168.0.1’ -- ‘The default password is ‘admin’’ or double click to run the program  ‘’

- Confirm whether the connection status is closed, If off, just turn on it.

- Check if the APN information is correct

- We can refer to this link:  https://youtu.be/N6LXeljdP3c to compare the APN information queried by inserting the sim card into the Android phone with the APN information identified by the device. If not,we can create a new profile to add the correct APN information, please refer to this link: https://youtu.be/0oUfrb1Fvww

- Check whether the flow limit is set

- Insert the SIM card into the mobile phone to test whether it get internet normally

- If you encountered other situations not described above, please update your router’s firmware or contact our tech support by send E-mail to support@tenda.com.cn